Modbus Protocol

Complete Technical Reference

& C++ Library Implementation

Protocol internals  •  Architecture  •  Implementation  •  Testing  •  Debugging

C++ Modbus Library — Project 4

PART I

The Modbus Protocol

# 1. History and Background

Modbus is a serial communication protocol originally developed by Modicon (now a brand of Schneider Electric) and published in 1979. It was created for use with their line of programmable logic controllers (PLCs). Because Modicon made the specification public and royalty-free from day one, it was adopted rapidly across the entire industrial automation industry.

Forty-five years later, Modbus remains the single most widely deployed industrial communication protocol on Earth. Estimates place the number of Modbus-capable devices currently in service at well over 100 million — meters, drives, sensors, controllers, and gateways from hundreds of manufacturers, all speaking the same 1979 specification.

## 1.1 Why Has It Survived?

Several properties have made Modbus uniquely durable:

- Extreme simplicity: The entire RTU frame specification fits on two pages. A working implementation can be written in a few hundred lines of C on an 8-bit microcontroller.

- Determinism: Request/response with strict timing rules. There is no ambiguity about when a transaction is complete.

- Vendor neutrality: No royalties, no certification fees, no mandatory conformance testing. Any company can implement it freely.

- Fault tolerance: CRC-16 error detection provides strong protection against single-bit and burst errors on noisy industrial wiring.

- Open specification: Freely downloadable from modbus.org. Every engineer can read the primary source.

## 1.2 Industrial Context

To understand why Modbus matters, consider what industrial communication looks like in practice. A factory floor might contain:

- 50 variable-frequency drives controlling motors, each reporting actual RPM and accepting speed setpoints

- 12 power meters measuring energy consumption on different circuit legs

- 8 temperature controllers on industrial ovens

- 24 digital I/O modules for limit switches, solenoids, and indicator lights

- A central SCADA (Supervisory Control and Data Acquisition) system that must read all of these devices every 100 milliseconds

Before fieldbus protocols like Modbus existed, each of these devices had a proprietary analog wiring interface — 4–20 mA current loops, 0–10 V voltage signals, or dry contact relays. Wiring a 50-drive system meant running hundreds of individual cables. Modbus replaced all of that with two wires (RS-485) or a single Ethernet cable (TCP), carrying structured digital data to and from all devices.

## 1.3 Physical Environments

Modbus was designed for harsh industrial environments:

| Environment | Challenge | Modbus Solution |
| --- | --- | --- |
| Factory floor | Electrical noise from motors, welders | RS-485 differential signalling rejects common-mode noise |
| Oil refinery | Long cable runs (hundreds of metres) | RS-485 supports up to 1,200 m |
| Power plant | High-voltage surroundings | Optical isolation of RS-485 transceivers |
| Smart building | Many devices, different vendors | Vendor-neutral protocol, standard register conventions |
| Solar farm | Remote monitoring over IP network | Modbus TCP over standard Ethernet / 4G routers |

# 2. The Modbus Data Model

Every Modbus server device exposes a standardised memory model. The client reads and writes to this model using function codes (operations). There are four distinct data tables, each accessible via different function codes.

## 2.1 The Four Data Tables

| Table Name | Data Type | Access | Legacy Address | PDU Address | Typical Use |
| --- | --- | --- | --- | --- | --- |
| Coils | 1-bit | Read / Write | 00001 – 09999 | 0x0000 – 0xFFFF | Relay outputs, motor ON/OFF, valve open/close |
| Discrete Inputs | 1-bit | Read only | 10001 – 19999 | 0x0000 – 0xFFFF | Limit switches, door sensors, digital inputs |
| Input Registers | 16-bit unsigned | Read only | 30001 – 39999 | 0x0000 – 0xFFFF | ADC readings, sensor values, counters |
| Holding Registers | 16-bit unsigned | Read / Write | 40001 – 49999 | 0x0000 – 0xFFFF | Setpoints, configuration, process variables |

## 2.2 Addressing: Legacy vs PDU

This is one of the most common sources of confusion when working with Modbus documentation. There are two addressing conventions:

### Legacy (Data Model) Addressing

Older documentation uses a 5-digit address with a type prefix. The first digit indicates the table:

- 0xxxx: Coils (e.g., 00001 = first coil)

- 1xxxx: Discrete Inputs (e.g., 10001 = first discrete input)

- 3xxxx: Input Registers (e.g., 30001 = first input register)

- 4xxxx: Holding Registers (e.g., 40001 = first holding register)

### PDU (Protocol Data Unit) Addressing

At the actual wire level — the bytes you put in the frame — addresses are zero-based integers within each table type. The table type is implied by the function code used. Therefore:

Legacy address 40001  →  PDU address 0x0000  (FC03 request)

Legacy address 40002  →  PDU address 0x0001  (FC03 request)

Legacy address 40100  →  PDU address 0x0063  (FC03 request)

Legacy address 00001  →  PDU address 0x0000  (FC01 request)

Formula:  PDU address = Legacy address - table_base - 1

Holding: PDU = (4xxxx) - 40001

Coil:    PDU = (0xxxx) - 00001

Rule: When writing protocol-level code (frame encoding), always use 0-based PDU addresses. When talking to users or reading device datasheets, you will see legacy addresses — convert before putting them in frames.

## 2.3 16-Bit Register Value Ranges

Holding registers and input registers are fundamentally 16-bit unsigned integers (0 to 65535). However, devices use them to represent many different real-world quantities through scaling:

| Physical Quantity | Raw Register Value | Scale Factor | Actual Value |
| --- | --- | --- | --- |
| Temperature | 253 | × 0.1 | 25.3 °C |
| Voltage | 2360 | × 0.1 | 236.0 V |
| Current | 157 | × 0.01 | 1.57 A |
| Motor Speed | 1500 | × 1 | 1500 RPM |
| Energy | 12450 | × 0.001 kWh | 12.450 kWh |
| Pressure | 2500 | × 0.1 bar | 250.0 bar |

32-bit values (float or 32-bit integer) require two consecutive 16-bit registers. The byte ordering of these two registers varies by manufacturer, creating four possible combinations:

| Word Order Name | Register[n] Content | Register[n+1] Content | Common In |
| --- | --- | --- | --- |
| Big-Endian (AB CD) | High word (bytes A,B) | Low word (bytes C,D) | Most common, Modbus default |
| Little-Endian (CD AB) | Low word (bytes C,D) | High word (bytes A,B) | Some older PLCs |
| Mid-Big (BA DC) | Byte-swapped high word | Byte-swapped low word | Rare |
| Mid-Little (DC BA) | Byte-swapped low word | Byte-swapped high word | Very rare |

# 3. The Client / Server (Master / Slave) Model

Modbus uses a strict master/slave (now called client/server) communication model. Understanding this model is essential before writing a single line of implementation code.

## 3.1 Rules of the Model

- Clients initiate everything: A client (master) sends a request. A server (slave) never sends data unsolicited. The server only responds to requests addressed to it.

- One speaker at a time (RTU): On a shared RS-485 bus, the bus is half-duplex. Only one device can drive the bus at a time. The master transmits a request, releases the bus, and waits for the addressed slave to respond.

- Request-response pairing: Every request produces exactly one response (unless it is a broadcast). There is no acknowledgement without a response.

- Broadcast (RTU only): A request sent to slave address 0x00 is a broadcast. All slaves execute the command but none of them respond. Used to write the same value to all devices simultaneously.

- TCP allows multiple sessions: In Modbus TCP, multiple clients can connect to a server simultaneously using separate TCP sessions. Each session is independent.

## 3.2 Transaction Flow (RTU)

CLIENT                              RS-485 BUS            SLAVE ID=1     SLAVE ID=2

│                                     │                     │              │

│── Request: [01 03 00 00 00 02 C4 0B] ──────────────────►  │              │

│                                     │   All slaves hear it │              │

│                                     │   ID=1 matches ──────┤              │

│                                     │   ID=2 ignores ──────┼──────────────┤

│                                                            │

│                         [Wait: inter-frame gap ~3.5ms]    │

│                                                            │

│◄── Response: [01 03 04 00 17 00 2B A9 CA] ────────────────┤

│                                                            │

│  Only slave ID=1 responds                                  │

## 3.3 Addressing Scheme

Each Modbus server device has a unique address (slave ID) configured on it — typically via DIP switches, a front-panel menu, or a configuration register.

| Slave ID Range | Purpose |
| --- | --- |
| 0x00 (0) | Broadcast address — all slaves execute, none respond |
| 0x01 – 0xF7 (1 – 247) | Unicast — only the addressed slave responds |
| 0xF8 – 0xFF (248 – 255) | Reserved — do not use |

Practical tip: RS-485 is limited to 32 "unit loads" per segment. With modern high-impedance transceivers (1/8 unit load), up to 256 devices can share one bus segment. Repeaters extend this further. In practice, most industrial installations have 8–32 devices per segment.

# 4. Modbus RTU — Frame Structure

RTU (Remote Terminal Unit) is the serial variant of Modbus. It transmits data as raw binary bytes with no start/end delimiters — framing is entirely time-based.

## 4.1 Complete Frame Layout

┌────────────┬────────────┬─────────────────────────┬────────────┬────────────┐

│  Slave ID  │    FC      │          Data           │  CRC Low   │  CRC High  │

│   1 byte   │  1 byte    │       variable          │   1 byte   │   1 byte   │

└────────────┴────────────┴─────────────────────────┴────────────┴────────────┘

◄─── Little-Endian ───►

◄────────────────────────── CRC calculated over these bytes ──────────────────►

| Field | Size | Range | Description |
| --- | --- | --- | --- |
| Slave ID | 1 byte | 0x00 – 0xFF | Device address. 0=broadcast, 1–247=unicast, 248–255=reserved. |
| Function Code (FC) | 1 byte | 0x01 – 0x7F | Operation type. Bit 7 set (0x80–0xFF) signals exception response. |
| Data | Variable | 0–252 bytes | Format depends on FC. Contains addresses, quantities, values. |
| CRC Low | 1 byte | 0x00 – 0xFF | Low byte of CRC-16. LITTLE-ENDIAN: low byte first. |
| CRC High | 1 byte | 0x00 – 0xFF | High byte of CRC-16. |

## 4.2 Frame Delimiting: The Silence Gap

Unlike many protocols, Modbus RTU does not use start/stop characters or length fields. Instead, frames are delimited by periods of bus silence.

- Inter-frame gap (≥ 3.5 char times): A silence of at least 3.5 character times marks the END of one frame and the START of a new one. "Character time" is the time needed to transmit one byte including start/stop/parity bits.

- Inter-character gap (< 1.5 char times): The gap between consecutive bytes within the SAME frame must be less than 1.5 character times. A gap longer than this means the frame has ended prematurely.

| Baud Rate | 1 Character Time | 1.5 Char Times (max within frame) | 3.5 Char Times (min between frames) |
| --- | --- | --- | --- |
| 1,200 | 8.33 ms | 12.50 ms | 29.17 ms |
| 9,600 | 1.04 ms | 1.56 ms | 3.65 ms |
| 19,200 | 0.52 ms | 0.78 ms | 1.82 ms |
| 38,400 | 0.26 ms | 0.39 ms | 0.91 ms |
| 115,200 | 0.087 ms | 0.130 ms | 0.304 ms |

Implementation note: At baud rates below 19,200, the minimum gap is large enough to implement with a hardware timer. At higher baud rates, the gap is only fractions of a millisecond — too short for many operating systems to time accurately. The Modbus specification allows implementations running at >19,200 baud to use fixed gaps of 1.75ms (inter-character) and 3.5ms (inter-frame) instead.

## 4.3 UART Configuration

For correct Modbus RTU communication, the UART must be configured as follows:

| Parameter | Value | Notes |
| --- | --- | --- |
| Mode | RAW binary | Never canonical/line mode — will corrupt binary data |
| Data bits | 8 | Defined by specification |
| Parity | Even (recommended) | Creates an 11-bit character frame: 1 start + 8 data + 1 parity + 1 stop |
| Parity (alternative) | None | Requires 2 stop bits for same 11-bit frame: 1+8+2 |
| Parity (alternative) | Odd | Also valid; less common in practice |
| Stop bits | 1 (with parity) | Use 2 stop bits only when parity = None |
| Flow control | None | Never use hardware or software flow control |

Why 11-bit characters? The inter-frame gap of 3.5 character times is defined in terms of 11-bit character frames. Using 8-bit characters would change the timing. The spec mandates 11-bit frames for this reason.

# 5. CRC-16 Error Detection

The CRC (Cyclic Redundancy Check) is Modbus RTU's only error detection mechanism. It is calculated over the entire frame content and appended as the last two bytes of every RTU frame.

## 5.1 Algorithm Specification

| Property | Value |
| --- | --- |
| Algorithm name | CRC-16/IBM (also called CRC-16/ARC or CRC-16-ANSI) |
| Polynomial | 0x8005 (normal form) / 0xA001 (reflected form) |
| Initial value | 0xFFFF |
| Input reflection | Yes (process LSB first) |
| Output reflection | Yes |
| Final XOR | 0x0000 (none) |
| Residue | 0x0000 (running CRC over full frame including CRC bytes gives 0) |
| Byte order | Little-endian: LOW byte appended FIRST, HIGH byte second |

## 5.2 Bit-by-Bit Algorithm

The reference implementation uses a bit-shifting loop. This is the most portable approach and uses only a 16-bit register:

uint16_t crc16_modbus(const uint8_t* data, size_t length) {

uint16_t crc = 0xFFFF;          // Step 1: initialise

for (size_t i = 0; i < length; i++) {

crc ^= (uint16_t)data[i];   // Step 2a: XOR byte into low byte of CRC

for (int j = 0; j < 8; j++) {    // Step 2b: process 8 bits

if (crc & 0x0001) {           // if LSB is 1

crc >>= 1;               // shift right

crc ^= 0xA001;           // XOR with reflected polynomial

} else {

crc >>= 1;               // just shift right

}

}

}

return crc;   // Step 3: return result

}

// To append to a frame (LITTLE-ENDIAN — low byte first):

frame.push_back(crc & 0xFF);          // CRC low byte

frame.push_back((crc >> 8) & 0xFF);   // CRC high byte

## 5.3 Lookup Table Algorithm (Faster)

For performance-critical implementations, precompute a 256-entry table. This reduces CRC calculation to one table lookup per byte instead of 8 iterations:

// Precompute table once at startup (or make it constexpr)

uint16_t crcTable[256];

for (int i = 0; i < 256; i++) {

uint16_t crc = (uint16_t)i;

for (int j = 0; j < 8; j++) {

crc = (crc & 1) ? (crc >> 1) ^ 0xA001 : (crc >> 1);

}

crcTable[i] = crc;

}

// Use the table:

uint16_t crc16_table(const uint8_t* data, size_t len) {

uint16_t crc = 0xFFFF;

for (size_t i = 0; i < len; i++) {

crc = (crc >> 8) ^ crcTable[(crc ^ data[i]) & 0xFF];

}

return crc;

}

## 5.4 Known Test Vectors

Use these vectors to validate your CRC implementation before any other testing:

| Input (hex bytes) | Expected CRC (hex) | Appended to frame as |
| --- | --- | --- |
| 01 03 00 00 00 02 | 0x0BC4 | [C4 0B] |
| 01 03 04 00 17 00 2B | 0x280A | [0A 28] |
| 01 06 00 00 05 DC | 0x038B | [8B 03] |
| 01 06 00 02 00 64 | 0xE129 | [29 E1] |
| 01 05 00 00 FF 00 | 0x3A8C | [8C 3A] |
| 01 05 00 00 00 00 | 0xCACD | [CD CA] |
| FF FF FF FF FF FF | 0x9401 | [01 94] |
| 00 00 00 00 | 0x2400 | [00 24] |
| 01 (single byte) | 0x807E | [7E 80] |

## 5.5 The Residue Property

Residue verification: Running the CRC algorithm over a complete valid frame — including the two appended CRC bytes — produces exactly 0x0000. This gives an alternative verification method: instead of extracting and comparing the CRC bytes, simply run the CRC over the entire received frame and check for a zero result.

uint8_t valid_frame[] = {0x01, 0x03, 0x00, 0x00, 0x00, 0x02, 0xC4, 0x0B};

uint16_t result = crc16_modbus(valid_frame, 8);

// result == 0x0000  ←  residue property confirmed

## 5.6 The Endianness Trap

This is the single most common Modbus implementation bug. All Modbus data fields (addresses, quantities, register values) are serialised BIG-ENDIAN (most significant byte first). The CRC is the ONE exception — it is appended LITTLE-ENDIAN (low byte first).

Frame data fields (BIG-ENDIAN):

Register value 0x05DC  →  byte sequence: [05 DC]  (high byte first)

Start address 0x0100   →  byte sequence: [01 00]  (high byte first)

CRC field (LITTLE-ENDIAN — the exception):

CRC value 0x0BC4       →  byte sequence: [C4 0B]  (LOW byte first)

← This is the OPPOSITE of how everything else is serialised →

Why? The original Modicon implementation stored the CRC in a 16-bit CPU register and wrote it to the serial port low-byte first (the natural byte order of the processor at the time). This became part of the specification and cannot be changed.

# 6. Function Codes — Complete Reference

Function codes (FCs) are the "operations" in the Modbus protocol. Each FC specifies what operation the server should perform and defines the exact byte layout of both the request and response frames.

## 6.1 Overview of Supported Function Codes

| FC (hex) | FC (dec) | Name | Operation | Max per request |
| --- | --- | --- | --- | --- |
| 0x01 | 1 | Read Coils | Read 1-bit outputs | 2000 coils |
| 0x02 | 2 | Read Discrete Inputs | Read 1-bit inputs | 2000 inputs |
| 0x03 | 3 | Read Holding Registers | Read 16-bit R/W registers | 125 registers |
| 0x04 | 4 | Read Input Registers | Read 16-bit R-only registers | 125 registers |
| 0x05 | 5 | Write Single Coil | Write one 1-bit output | 1 coil |
| 0x06 | 6 | Write Single Register | Write one 16-bit register | 1 register |
| 0x08 | 8 | Diagnostics | Loopback, counters | varies |
| 0x0F | 15 | Write Multiple Coils | Write multiple 1-bit outputs | 1968 coils |
| 0x10 | 16 | Write Multiple Registers | Write multiple 16-bit registers | 123 registers |
| 0x17 | 23 | R/W Multiple Registers | Read and write in one transaction | 125 read / 121 write |

## 6.2 FC 0x03 — Read Holding Registers

This is the most important and most widely used function code. It reads one to 125 consecutive 16-bit holding registers from the server.

### Request Frame (always 8 bytes for FC03)

Byte:  [  0  ] [  1  ] [  2  ] [  3  ] [  4  ] [  5  ] [  6  ] [  7  ]

┌──────┬───────┬───────┬───────┬───────┬───────┬───────┬───────┐

│  01  │  03   │  00   │  00   │  00   │  02   │  C4   │  0B  │

└──────┴───────┴───────┴───────┴───────┴───────┴───────┴───────┘

SlaveID  FC     StartAddr (Hi) (Lo)  Quantity (Hi) (Lo)  CRC (Lo)(Hi)

SlaveID    = 0x01      Target device address

FC         = 0x03      Read Holding Registers

Start Addr = 0x0000    First register address (0-based)

Quantity   = 0x0002    Number of registers to read (1–125)

CRC        = 0xC4,0x0B CRC low byte first (little-endian)

### Response Frame

Byte:  [  0  ] [  1  ] [  2  ] [  3  ] [  4  ] [  5  ] [  6  ] [  7  ] [  8  ]

┌──────┬───────┬───────┬───────┬───────┬───────┬───────┬───────┬───────┐

│  01  │  03   │  04   │  00   │  17   │  00   │  2B   │  0A   │  28  │

└──────┴───────┴───────┴───────┴───────┴───────┴───────┴───────┴───────┘

SlaveID  FC    ByteCnt  Reg[0] (Hi)(Lo)  Reg[1] (Hi)(Lo)  CRC(Lo)(Hi)

ByteCount  = 0x04      = Quantity × 2 = 2 × 2 = 4

Reg[0]     = 0x0017    = 23

Reg[1]     = 0x002B    = 43

CRC        = 0x0A,0x28 Low byte first

### Server-Side Validation Rules

- Quantity: Must be 1 to 125. If 0 or > 125: return exception 0x03 (Illegal Data Value).

- Address range: startAddr + quantity - 1 must not exceed the last valid register address. If so: return exception 0x02 (Illegal Data Address).

- Byte count in response: Must always equal quantity × 2.

## 6.3 FC 0x06 — Write Single Register

Writes a single 16-bit value to one holding register. The server echoes back the entire request frame as confirmation.

Request:   01  06  00 02  05 DC  2A C3

ID  FC  Addr  Value   CRC

Write value 0x05DC (1500) to register 0x0002

Response:  01  06  00 02  05 DC  2A C3     ← identical echo

Echo validation: The client must verify that the response matches the request byte-for-byte. Any difference indicates an error.

## 6.4 FC 0x05 — Write Single Coil

Writes a 1-bit coil output. The value is not 0 or 1 — it uses a special 16-bit encoding:

| Desired State | Value High Byte | Value Low Byte | Combined 16-bit Value |
| --- | --- | --- | --- |
| ON | 0xFF | 0x00 | 0xFF00 |
| OFF | 0x00 | 0x00 | 0x0000 |
| Any other value | — | — | Server returns exception 0x03 (Illegal Data Value) |

Turn coil 0x0000 ON:   01  05  00 00  FF 00  8C 3A

Turn coil 0x0000 OFF:  01  05  00 00  00 00  CD CA

Both produce an echo response identical to the request.

## 6.5 FC 0x01 — Read Coils

Reads up to 2000 coil (1-bit) values. The coil bits are packed into bytes, eight coils per byte, least significant bit first.

Request:  01  01  00 00  00 0A  BC 0D

ID  FC  StartAddr=0  Qty=10 (read 10 coils)  CRC

Response: 01  01  02  CD  01  [CRC]

ID  FC  BC=2  Byte0  Byte1

Byte0 = 0xCD = 1100 1101

7654 3210  ← bit positions (coil addresses)

Bit 0 (addr 0x0000) = 1  → coil ON

Bit 1 (addr 0x0001) = 0  → coil OFF

Bit 2 (addr 0x0002) = 1  → coil ON

Bit 3 (addr 0x0003) = 1  → coil ON

...

Byte1 = 0x01 = 0000 0001

Bit 0 (addr 0x0008) = 1  → coil ON

Bit 1 (addr 0x0009) = 0  → coil OFF

ByteCount = ceil(10 / 8) = 2  (last byte zero-padded)

## 6.6 FC 0x10 — Write Multiple Registers

Writes 1 to 123 consecutive holding registers in a single transaction. More efficient than multiple FC06 writes.

Request:  01  10  00 00  00 02  04  00 64  00 C8  [CRC]

ID  FC  Addr   Qty   BC  Reg[0]  Reg[1]

Addr = 0x0000  Start address

Qty  = 0x0002  Number of registers (2)

BC   = 0x04    Byte count = Qty × 2 = 4

Reg[0] = 0x0064 = 100

Reg[1] = 0x00C8 = 200

Response:  01  10  00 00  00 02  [CRC]

ID  FC  Addr   Qty   (echo of address and quantity only)

## 6.7 FC 0x0F — Write Multiple Coils

Writes 1 to 1968 coils. Coils are packed into bytes (8 per byte, LSB first) just like the read response.

Request:  01  0F  00 00  00 03  01  05  [CRC]

ID  FC  Addr   Qty   BC  CoilByte

Qty  = 0x0003  Write 3 coils

BC   = 0x01    ByteCount = ceil(3/8) = 1

CoilByte = 0x05 = 0000 0101

Bit 0 (coil 0) = 1  → ON

Bit 1 (coil 1) = 0  → OFF

Bit 2 (coil 2) = 1  → ON

Response:  01  0F  00 00  00 03  [CRC]

ID  FC  Addr   Qty   (echo of address and quantity)

## 6.8 FC 0x04 and FC 0x02 — Read Input Registers and Discrete Inputs

These function codes are structurally identical to FC03 and FC01 respectively, but operate on read-only data tables:

- FC 0x02 (Read Discrete Inputs): Identical byte structure to FC01 (Read Coils). Bit-packed response. Reads from the discrete input table — values that the server sets internally, not writable by clients.

- FC 0x04 (Read Input Registers): Identical byte structure to FC03 (Read Holding Registers). 16-bit values. Reads from the input register table — sensor measurements or calculated values, not writable by clients.

# 7. Exception Responses

When a Modbus server receives a valid (CRC-correct) request it cannot fulfil, it sends an exception response rather than a normal response. This is fundamentally different from a communication error — an exception is a deliberate rejection.

## 7.1 Exception Frame Format

Normal response:    [SlaveID] [FC]       [Data...]  [CRC Lo] [CRC Hi]

Exception response: [SlaveID] [FC | 0x80] [ExCode]   [CRC Lo] [CRC Hi]

Detection rule:  if (receivedFC & 0x80) != 0  →  exception response

Example:

Request:   01  03  00 64  00 01  [CRC]   ← read reg at addr 100

Exception: 01  83  02        [CRC]

ID  FC|80  IllegalDataAddress

0x83 = 0x03 | 0x80  ← original FC was 0x03

0x02 = exception code: Illegal Data Address

## 7.2 Standard Exception Codes

| Code | Hex | Name | Meaning | When to Return |
| --- | --- | --- | --- | --- |
| 1 | 0x01 | Illegal Function | FC not supported by this server | Received FC is not in your dispatch table |
| 2 | 0x02 | Illegal Data Address | Register/coil address does not exist | Address or address+quantity-1 beyond valid range |
| 3 | 0x03 | Illegal Data Value | Value in data field is not acceptable | Quantity = 0, quantity > max, coil value ≠ 0xFF00/0x0000 |
| 4 | 0x04 | Slave Device Failure | Unrecoverable error in the server | Hardware fault, exception caught in handler |
| 5 | 0x05 | Acknowledge | Request accepted, processing takes time | Long-duration operations (optional) |
| 6 | 0x06 | Slave Device Busy | Currently processing another request | Busy state (optional) |
| 8 | 0x08 | Memory Parity Error | Detected parity error in memory | EEPROM/Flash read error (optional) |

## 7.3 Client Handling Rules

- Do NOT retry exceptions: A Modbus exception is a definitive answer. The server received the request perfectly and deliberately rejected it. Retrying will produce the same exception.

- DO retry timeouts and CRC errors: These indicate communication failures, not protocol rejections. Up to 3 retries are typical.

- Distinguish error types in your API: Use different error types for transport failures, protocol errors, and Modbus exceptions so the caller can handle them appropriately.

# 8. Modbus TCP

Modbus TCP maps the Modbus application protocol onto standard TCP/IP, replacing the serial-specific framing and CRC with a network-friendly header. The application layer (function codes, register model) is identical to RTU.

## 8.1 Differences from RTU

| Property | Modbus RTU | Modbus TCP |
| --- | --- | --- |
| Physical transport | RS-485 / RS-232 serial | Ethernet, Wi-Fi, or any TCP/IP network |
| Frame delimiter | Silence gap (3.5 char times) | MBAP Length field |
| Error detection | CRC-16 (2 bytes) | TCP/IP checksum + MBAP |
| Address field | 1-byte Slave ID in frame | 1-byte Unit ID in MBAP header |
| Standard port | N/A (serial port) | TCP port 502 |
| Concurrent clients | 1 (half-duplex bus) | Many (separate TCP sessions) |
| Typical latency | <10ms (deterministic) | Network-dependent, usually <100ms |
| Transaction matching | Not needed (sequential) | Transaction ID for pipelining |

## 8.2 MBAP Header Structure

MBAP stands for Modbus Application Protocol. Every Modbus TCP frame starts with a 7-byte MBAP header:

Byte:  [0] [1]   [2] [3]   [4] [5]   [6]   [7]  [8..N]

┌───────┬───────┬───────┬─────┬──────────────────┐

│Trans  │Proto  │Length │Unit │       PDU        │

│  ID   │  ID   │       │ ID  │  (FC + Data)     │

└───────┴───────┴───────┴─────┴──────────────────┘

◄──────── MBAP Header (6 bytes) ──────►

Transaction ID:  2 bytes, big-endian. Client-assigned counter.

Server echoes this value unchanged.

Allows matching responses to requests when pipelining.

Protocol ID:     2 bytes, always 0x0000 for Modbus.

Length:          2 bytes, big-endian.

= bytes remaining after this field

= 1 (Unit ID) + 1 (FC) + len(Data)

Unit ID:         1 byte. Equivalent to Slave ID in RTU.

Used when a TCP gateway routes to downstream RTU devices.

Set to 0xFF or 0x00 for direct TCP connections.

## 8.3 TCP Frame Example

Request: Read 2 holding registers at address 0x0000 from Unit ID 1

Bytes:  00 01  00 00  00 06  01  03  00 00  00 02

─────  ─────  ─────  ──  ──  ─────  ─────

TrID   Proto  Len   UID  FC  Addr   Qty

Length = 0x0006 = 6 = UID(1) + FC(1) + Addr(2) + Qty(2)

Response:

Bytes:  00 01  00 00  00 07  01  03  04  00 17  00 2B

TrID   Proto  Len   UID  FC  BC  Reg0   Reg1

Length = 0x0007 = 7 = UID(1) + FC(1) + BC(1) + Data(4)

No CRC appended — TCP checksum handles integrity.

## 8.4 Receiving a Complete TCP Frame

Since TCP is a stream protocol, bytes may arrive in fragments. A robust receive function must:

- Read exactly 6 bytes — the fixed-size MBAP header.

- Extract the Length field from bytes [4..5] of the MBAP header.

- Read exactly Length more bytes — the PDU payload.

- Validate Protocol ID = 0x0000. Reject if anything else.

Never assume all bytes arrive in one read() call. TCP may split them across multiple packets. Always loop until the expected byte count is accumulated.

# 9. RS-485 Physical Layer

RS-485 is the electrical standard most commonly used as the physical layer for Modbus RTU. Understanding it is essential for debugging field problems.

## 9.1 Differential Signalling

Device A                   RS-485 Bus                  Device B

┌──────────┐                                          ┌──────────┐

│ UART TX  ├── RS-485 ──► A+ ──────────────────────►├── RS-485 ──► UART RX │

│ UART RX  ├── RS-485 ──► B- ──────────────────────►├── RS-485 ──► UART RX │

└──────────┘                                          └──────────┘

Signal = A - B

If A - B > +200mV  →  logic 1

If A - B < -200mV  →  logic 0

Common-mode noise affects A and B equally → cancels out in the differential

This differential signalling is why RS-485 is so robust in electrically noisy environments like factory floors. A 500V transient spike on both lines simultaneously produces no differential signal and is completely rejected.

## 9.2 Key RS-485 Parameters

| Parameter | Value | Notes |
| --- | --- | --- |
| Max cable length | 1,200 m at 100 kbps | Inversely proportional to baud rate |
| Max devices | 32 unit loads per segment | More with 1/8 UL (high-impedance) transceivers |
| Topology | Daisy-chain / linear bus | Star or stub connections cause reflections |
| Termination | 120 Ω at each end | Match cable impedance; only needed >300m or >19200 baud |
| Common mode range | -7V to +12V | Provides isolation from ground differences between buildings |
| Idle state | Bus at logic 1 (A > B) | Fail-safe resistors hold bus idle when no driver active |
| Direction control | DE/RE pins on transceiver | Master controls bus direction; half-duplex operation |

## 9.3 Half-Duplex Operation

RS-485 is half-duplex: only one device can transmit at a time. Each transceiver chip has Driver Enable (DE) and Receiver Enable (~RE) pins:

- To transmit: DE = HIGH, ~RE = HIGH. This enables the driver and disables the receiver.

- To receive: DE = LOW, ~RE = LOW. This disables the driver and enables the receiver.

On PC USB-to-RS485 adapters, this direction switching is handled automatically by the adapter firmware (often using a timer or data-pattern detection). On bare microcontrollers, you must manually toggle a GPIO pin connected to DE/~RE before and after each transmission — this is the RS-485 "direction control" often seen in embedded firmware.

PART II

C++ Library Architecture

# 10. Design Goals and Principles

The library is designed around six architectural goals that directly reflect the project requirements and the constraints of industrial embedded systems:

| Goal | Consequence |
| --- | --- |
| Separation of concerns | Each module does exactly one thing. CrcEngine only computes CRC. RegisterMap only stores values. |
| Testability without hardware | IModbusTransport is a pure abstract interface. MockTransport enables 100% of logic to be tested with no serial port. |
| Portability | OS-specific code is confined to *_Linux.cpp / *_Windows.cpp files. Public headers contain no platform-specific symbols. |
| Typed error handling | std::expected<T, ModbusError> forces callers to handle errors. No silent failure paths. |
| Modern C++ idioms | C++23 features: std::expected, std::span, constexpr, range-based for, RAII throughout. |
| Zero-cost abstractions where possible | std::span is non-allocating. CrcEngine is stateless. Codec functions are pure transformations. |

## 10.1 SOLID Principles Applied

| Principle | Application in This Library |
| --- | --- |
| Single Responsibility | CrcEngine: only CRC. ModbusFrameCodec: only encoding/decoding. RegisterMap: only data storage. Each class has one reason to change. |
| Open/Closed | New transport (Rs485Transport) added without modifying ModbusClient. New FC handler added without changing other handlers. |
| Liskov Substitution | TcpTransport, UartTransport, and MockTransport are all interchangeable wherever IModbusTransport* is expected. |
| Interface Segregation | IModbusTransport has 5 methods only: send, receive, flush, isOpen, close. No transport is forced to implement irrelevant functionality. |
| Dependency Inversion | ModbusClient and ModbusServer depend on IModbusTransport (abstraction). Never on UartTransport (concretion). Transport injected via constructor. |

# 11. Layer Architecture

╔══════════════════════════════════════════════════════════════════╗

║                     APPLICATION LAYER                           ║

║             (Demo Client / Demo Server programs)                ║

╠══════════════════════════════════════════════════════════════════╣

║                    LIBRARY PUBLIC API                           ║

║  ┌─────────────────────────┐  ┌──────────────────────────────┐  ║

║  │     ModbusClient        │  │       ModbusServer           │  ║

║  │ readHoldingRegisters()  │  │  start() / stop()            │  ║

║  │ writeSingleRegister()   │  │  processOneRequest()         │  ║

║  │ readCoils()  ...        │  │                              │  ║

║  └──────────┬──────────────┘  └────────────┬─────────────────┘  ║

╠═════════════╪════════════════════════════════╪══════════════════╣

║             │      FRAME CODEC LAYER          │                 ║

║  ┌──────────▼────────────────────────────────▼─────────────┐   ║

║  │                ModbusFrameCodec                          │   ║

║  │  encodeRtuRequest()     decodeRtuResponse()              │   ║

║  │  encodeTcpRequest()     decodeTcpResponse()              │   ║

║  └──────────────────────────────┬──────────────────────────┘   ║

╠════════════════════════════════╪═══════════════════════════════╣

║          CRC ENGINE            │                               ║

║  ┌─────────────────────────────▼───────────────────────────┐   ║

║  │    CrcEngine: calculate(data, len) / verify(frame, len)  │   ║

║  └──────────────────────────────────────────────────────────┘   ║

╠════════════════════════════════════════════════════════════════╣

║             TRANSPORT ABSTRACTION LAYER                        ║

║  ┌──────────────────────────────────────────────────────────┐  ║

║  │          IModbusTransport (pure abstract interface)       │  ║

║  │    send() / receive() / flush() / isOpen() / close()     │  ║

║  └────────────┬──────────────────────────┬──────────────────┘  ║

║               │                          │                     ║

║  ┌────────────▼───────┐      ┌───────────▼──────────────────┐  ║

║  │   UartTransport    │      │      TcpTransport            │  ║

║  │  (Linux/Windows)   │      │  (POSIX sockets)             │  ║

║  └────────────────────┘      └──────────────────────────────┘  ║

╚════════════════════════════════════════════════════════════════╝

## 11.1 Data Flow: Client Read Request

user code

│  client.readHoldingRegisters(slaveId=1, addr=0, count=2)

▼

ModbusClient::readHoldingRegisters()

│  Validates: count ∈ [1,125]

│  Creates ModbusFrame {slaveId=1, FC=0x03, data=[00 00 00 02]}

▼

ModbusClient::sendAndReceive(frame)

│  transport_->flush()                  ← clear stale bytes

│  bytes = ModbusFrameCodec::encodeRtuRequest(frame)

│  → [01 03 00 00 00 02 C4 0B]

│  transport_->send(bytes)

│  rawResp = transport_->receive(256, 1000ms)

│  frame = ModbusFrameCodec::decodeRtuResponse(rawResp)

│  → verifies CRC, extracts slaveId, FC, data

│  validates: frame.slaveId == 1, FC == 0x03

▼

ModbusClient::readHoldingRegisters()  (cont.)

│  Extracts register values from frame.data

│  reg[0] = (data[1]<<8)|data[2] = 0x0017 = 23

│  reg[1] = (data[3]<<8)|data[4] = 0x002B = 43

▼

user code  ← receives std::vector<uint16_t>{23, 43}

## 11.2 Data Flow: Server Request Handling

ModbusServer worker thread

│  transport_->receive(256, 100ms)

│  → [01 03 00 00 00 02 C4 0B]

│  ModbusFrameCodec::decodeRtuResponse(bytes)

│  → verifies CRC ✓, slaveId=1, FC=0x03

│  frame.slaveId == slaveId_ ?  yes → process

▼

ModbusServer::handleRequest(frame)

│  switch(FC=0x03) → handleReadHoldingRegisters()

▼

handleReadHoldingRegisters()

│  startAddr = 0x0000, quantity = 2

│  validate quantity ∈ [1,125] ✓

│  validate address range ✓

│  for i in 0..1: registerMap_->readHoldingRegister(i)

│  build response: [byteCount=4, 00,17, 00,2B]

▼

ModbusFrameCodec::encodeRtuRequest(response)

│  → [01 03 04 00 17 00 2B 0A 28]

▼

transport_->send(responseBytes)

PART III

C++ Implementation Details

# 12. Type System and Error Handling

## 12.1 Type Aliases

The library uses type aliases (not preprocessor macros) for fixed-width integer types:

using u8  = uint8_t;   // 1-byte unsigned

using u16 = uint16_t;  // 2-byte unsigned (registers, addresses)

using u32 = uint32_t;  // 4-byte unsigned

using sz  = size_t;    // platform-native size type

Why not #define? Preprocessor macros have no scope. A #define u8 in a header file would silently rename "u8" everywhere across the entire codebase, including in third-party libraries and system headers. Using types respects C++ scope rules and enables proper overload resolution.

## 12.2 Error Type Hierarchy

All public API methods return std::expected<T, ModbusError>. ModbusError is a std::variant containing one of three error categories:

using ModbusError = std::variant<

TransportErrorCode,    // communication failures

ProtocolErrorCode,     // frame-level protocol errors

ModbusException        // server deliberately rejected request

>;

enum class TransportErrorCode {

Timeout,           // No response within timeout period

ConnectionFailed,  // Cannot open port / connect socket

ReadError,         // I/O error during receive

WriteError,        // I/O error during send

};

enum class ProtocolErrorCode {

CrcMismatch,           // Received CRC does not match calculated

InvalidFrameLength,    // Frame too short or inconsistent

SlaveIdMismatch,       // Response from wrong device

UnexpectedResponse,    // FC in response != FC in request

};

struct ModbusException {

u8 functionCode;   // Original FC (without bit 7)

u8 exceptionCode;  // 0x01, 0x02, 0x03, or 0x04

std::string description() const;

};

## 12.3 Using std::expected

std::expected<T, E> holds either a value of type T or an error of type E — never both. It forces the caller to explicitly handle the error case:

auto result = client.readHoldingRegisters(1, 0x0000, 3);

if (result.has_value()) {

// Success path

for (u16 val : *result) {

std::cout << val << "\n";

}

} else {

// Error path — must handle

std::visit([](auto&& e) {

using T = std::decay_t<decltype(e)>;

if constexpr (std::is_same_v<T, TransportErrorCode>) {

std::cerr << "Transport error\n";

} else if constexpr (std::is_same_v<T, ProtocolErrorCode>) {

std::cerr << "Protocol error\n";

} else {

std::cerr << e.description() << "\n";

}

}, result.error());

}

## 12.4 The sendAndReceive Algorithm

This private method is the heart of the client. Every public API method calls it. It implements the complete request-response cycle with retry logic:

Expected<ModbusFrame, ModbusError> sendAndReceive(const ModbusFrame& request) {

u8 attempts = 0;

while (true) {

// 1. Flush stale bytes from previous failed transactions

transport_->flush();

// 2. Encode and send

auto encoded = ModbusFrameCodec::encodeRtuRequest(request);

auto sendResult = transport_->send(encoded);

if (!sendResult) return Unexpected(sendResult.error());  // not retryable

// 3. Receive

auto recvResult = transport_->receive(256, timeout_);

if (!recvResult) {

if (isTimeout(recvResult.error())) {

if (++attempts >= retryCount_) return Unexpected(Timeout);

continue;  // ← retry

}

return Unexpected(recvResult.error());

}

// 4. Decode (verifies CRC)

auto decoded = ModbusFrameCodec::decodeRtuResponse(recvResult.value());

if (!decoded) {

if (isCrcMismatch(decoded.error())) {

if (++attempts >= retryCount_) return Unexpected(CrcMismatch);

continue;  // ← retry

}

return Unexpected(decoded.error());

}

// 5. Exception response — never retry

if (decoded->isExceptionResponse()) {

ModbusException ex{decoded->functionCode & 0x7F,

decoded->exceptionCode()};

return Unexpected(ex);

}

// 6. Validate slave ID

if (decoded->slaveID != request.slaveID) {

if (++attempts >= retryCount_) return Unexpected(SlaveIdMismatch);

continue;

}

return *decoded;  // success

}

}

# 13. RegisterMap — Thread-Safe Data Store

RegisterMap is the server's Modbus memory. It is used by both the server thread (reading/writing via Modbus client commands) and the application thread (updating sensor values, reading setpoints).

## 13.1 Thread Safety Design

// Naive (WRONG) — callback deadlock:

void writeHoldingRegister(u16 addr, u16 val) {

std::lock_guard lk(mutex_);

holdingRegisters_[addr] = val;

if (auto cb = callbacks_[addr]) cb(addr, val);  // ← DEADLOCK

// cb tries to call readHoldingRegister() which also locks mutex_

}

// Fixed (CORRECT) — release lock before callback:

void writeHoldingRegister(u16 addr, u16 val) {

HoldingRegWriteCallback cb;

{

std::lock_guard lk(mutex_);

holdingRegisters_[addr] = val;

auto it = holdingWriteCBs_.find(addr);

if (it != holdingWriteCBs_.end()) cb = it->second;

}  // ← mutex released here

if (cb) cb(addr, val);  // ← safe: lock not held

}

## 13.2 Compile-Time Register Layout

The server's register layout can be defined at compile time using constexpr data structures. This gives named access and build-time documentation:

// server/register_map.h

struct RegisterDescriptor {

u16             address;

std::string_view name;

std::string_view unit;

float           scale;    // real_value = raw * scale

bool            writable; // client-writable?

};

constexpr std::array<RegisterDescriptor, 4> HOLDING_REGISTERS = {{

{0x0000, "Motor Speed",          "RPM",  1.0f,  false},

{0x0001, "Temperature",          "C",    0.1f,  false},

{0x0002, "Temperature Setpoint", "C",    0.1f,  true },

{0x0003, "Fault Code",           "",     1.0f,  false},

}};

// Valid address lookup (constexpr — evaluated at compile time for constants)

constexpr bool isValidHolding(u16 addr) {

for (const auto& r : HOLDING_REGISTERS)

if (r.address == addr) return true;

return false;

}

# 14. Transport Layer Implementation

## 14.1 Linux UART — termios Configuration

On Linux, serial ports are configured through the POSIX termios API. The critical settings for Modbus RTU:

// Opening the port

int fd = open("/dev/ttyUSB0", O_RDWR | O_NOCTTY | O_NONBLOCK);

// Get current settings

struct termios tty{};

tcgetattr(fd, &tty);

// RAW MODE — essential for binary data

// Disable all input/output processing

tty.c_lflag &= ~(ICANON|ECHO|ECHOE|ISIG|IEXTEN);

tty.c_iflag &= ~(IXON|IXOFF|IXANY|ICRNL|INLCR|BRKINT|PARMRK|ISTRIP);

tty.c_oflag &= ~(OPOST|ONLCR);

// Baud rate

cfsetispeed(&tty, B9600);

cfsetospeed(&tty, B9600);

// 8 data bits, no parity, 1 stop bit

tty.c_cflag &= ~CSIZE; tty.c_cflag |= CS8;

tty.c_cflag &= ~PARENB;

tty.c_cflag &= ~CSTOPB;

tty.c_cflag |= CREAD | CLOCAL;

// Non-blocking reads (we use select() for timeout)

tty.c_cc[VMIN] = 0; tty.c_cc[VTIME] = 0;

tcsetattr(fd, TCSANOW, &tty);

## 14.2 Receive with select() Timeout

The receive function uses select() for accurate timeout handling and detects the RTU inter-frame gap:

std::vector<u8> buf;

auto deadline = std::chrono::steady_clock::now() + timeout;

while (buf.size() < maxLength) {

// Wait up to remaining time or 5ms (inter-frame gap detector)

auto remaining = duration_cast<microseconds>(deadline - now());

long wait_us = min(remaining.count(), 5000LL);

struct timeval tv{ wait_us/1000000, wait_us%1000000 };

fd_set rset; FD_ZERO(&rset); FD_SET(fd_, &rset);

int ready = select(fd_+1, &rset, nullptr, nullptr, &tv);

if (ready == 0) {

if (!buf.empty()) break;  // 5ms silence → frame complete

if (now() >= deadline) return Unexpected(Timeout);

} else if (ready > 0) {

u8 byte; read(fd_, &byte, 1);

buf.push_back(byte);

}

}

if (buf.empty()) return Unexpected(Timeout);

return buf;

PART IV

Bug Analysis and Fixes

# 15. The 10 Critical Bugs

The original codebase contained 10 significant bugs ranging from silent data corruption to complete feature absence. Each is documented here with root cause analysis, the exact code that was wrong, and the fix applied.

## Bug #1 — Exception Response Never Detected

| Property | Detail |
| --- | --- |
| Location | src/ModbusFrameCodec.cpp — decodeRtuResponse() |
| Severity | Critical — Modbus exception responses from the server were silently treated as normal frames |
| Root Cause | frame.isExceptionResponse() was called BEFORE frame.functionCode was assigned |

// BUGGY CODE:

ModbusFrame frame;

frame.slaveID = buffer[0];

// functionCode NOT YET ASSIGNED — still 0 from default init

if (frame.isExceptionResponse()) {   // (0 & 0x80) = 0 → ALWAYS FALSE

// This code was NEVER reached

}

frame.functionCode = buffer[1];   // assigned AFTER the check

// FIX: assign functionCode BEFORE the check

frame.functionCode = buffer[1];

if (frame.isExceptionResponse()) {   // now works correctly

// ...

}

## Bug #2 — sendAndReceive: Broken Retry Loop

| Property | Detail |
| --- | --- |
| Location | include/modbus/ModbusClient.h — sendAndReceive() |
| Severity | Critical — retry logic non-functional; undefined behaviour on timeout |
| Root Cause | setRetryCount(0) at start wiped the configuration; no continue statements; fell through to .value() on empty expected |

// BUGGY CODE (simplified):

std::expected<ModbusFrame, ModbusError> sendAndReceive(const ModbusFrame& req) {

setRetryCount(0);   // ← OVERWRITES USER CONFIG permanently!

while (true) {

// ... send ...

auto recv = transport_->receive(...);

if (!recv) {

if (isTimeout()) {

if (++retryCount_ > 3) return Unexpected(Timeout);

// NO continue HERE → falls through!

}

}

auto decoded = codec::decode(recv.value());  // UB if recv failed!

}

}

// FIX: use a LOCAL attempts counter; always continue on retry

u8 attempts = 0;

while (true) {

// ...

if (isTimeout(recv.error())) {

if (++attempts >= retryCount_) return Unexpected(Timeout);

continue;   // ← must have this

}

}

## Bug #3 — writeSingleCoil Always Sends OFF

| Property | Detail |
| --- | --- |
| Location | src/ModbusClient.cpp — writeSingleCoil() |
| Severity | Critical — coil write always sent 0x0000 (OFF) regardless of requested state |
| Root Cause | After correctly building the frame, code overwrote request.data using (value >> 8) where value is bool. true >> 8 = 0 always. |

// BUGGY CODE:

ModbusFrame request = codec::makeWriteSingleCoilRequest(slaveId, addr, value);

// ^ this correctly sets data = [addr_hi, addr_lo, 0xFF, 0x00] for ON

request.data = {

(addr >> 8) & 0xFF, addr & 0xFF,

(value >> 8) & 0xFF,   // value is bool! true>>8 = 0 ALWAYS

value & 0xFF           // true&0xFF = 1 but ON must be 0xFF, not 0x01

};

// Result: always sends [addr_hi, addr_lo, 0x00, 0x01] which is INVALID

// FIX: simply use the frame as-is without overwriting

ModbusFrame request = codec::makeWriteSingleCoilRequest(slaveId, addr, value);

// Do NOT modify request.data

## Bug #4 — registerValue() Wrong Byte Offsets

| Property | Detail |
| --- | --- |
| Location | include/modbus/ModbusFrame.h — registerValue() |
| Severity | High — extracted register values were wrong; first register read the byte-count as high byte |
| Root Cause | FC03 response data layout: [byteCount, hi0, lo0, hi1, lo1, ...]. Code used data[0+i*2] which for i=0 reads data[0]=byteCount, not the register high byte. |

// BUGGY CODE:

u16 registerValue(sz i) const {

sz offset = 1 + (i * 2);

return (data[0 + i * 2] << 8) | data[2 + i * 2];

//      ↑ for i=0: data[0] = byteCount, NOT high byte!

}

// FIX:

u16 registerValue(sz i) const {

sz offset = 1u + i * 2u;   // skip data[0] (byte count)

if (offset + 1 >= data.size()) return 0;

return (static_cast<u16>(data[offset]) << 8) | data[offset + 1];

}

// FC03 response data layout:

// data[0]      = byteCount (e.g., 4 for 2 registers)

// data[1..2]   = register[0] (high byte, low byte)

// data[3..4]   = register[1] (high byte, low byte)

## Bug #5 — makeWriteMultipleCoilsRequest: Missing Byte-Count and Coil Data

| Property | Detail |
| --- | --- |
| Location | src/ModbusFrameCodec.cpp |
| Severity | Critical — FC0F requests were 4 bytes short; all coil values lost in transmission |
| Root Cause | Function built only the 4-byte header (startAddr + count). The byteCount byte and the packed coil bit data were never added. |

// BUGGY CODE (only header, no coil data):

frame.data.push_back((startAddr >> 8) & 0xFF);

frame.data.push_back(startAddr & 0xFF);

frame.data.push_back((count >> 8) & 0xFF);

frame.data.push_back(count & 0xFF);

// Missing: byteCount byte and packed coil bits!

// FIX: add byteCount and LSB-first bit-packed coil data

u8 byteCount = (count + 7) / 8;

frame.data.push_back(byteCount);

for (u8 i = 0; i < byteCount; i++) {

u8 byte = 0;

for (int bit = 0; bit < 8; bit++) {

size_t idx = i * 8 + bit;

if (idx < values.size() && values[idx])

byte |= (1u << bit);   // LSB first

}

frame.data.push_back(byte);

}

## Bug #6 — makeWriteMultipleRegistersRequest: Missing Register Data

| Property | Detail |
| --- | --- |
| Location | src/ModbusFrameCodec.cpp |
| Severity | Critical — FC10 requests contained no register values; server received garbage data |
| Root Cause | Same pattern as Bug #5 — only the 4-byte header was built. byteCount and all register values omitted. |

// BUGGY CODE:

frame.data.push_back((startAddr >> 8) & 0xFF);

frame.data.push_back(startAddr & 0xFF);

frame.data.push_back((count >> 8) & 0xFF);

frame.data.push_back(count & 0xFF);

// Missing: byteCount + actual register values!

// FIX:

u8 byteCount = count * 2;

frame.data.push_back(byteCount);

for (u16 val : values) {

frame.data.push_back((val >> 8) & 0xFF);  // big-endian

frame.data.push_back(val & 0xFF);

}

## Bug #7 — All 8 Server FC Handlers: Never Implemented

| Property | Detail |
| --- | --- |
| Location | src/ModbusServer.cpp |
| Severity | Critical — any attempt to use the server produced a linker error (undefined symbols) |
| Root Cause | All 8 handler methods were declared in the header but had zero implementation in the .cpp file. The file was effectively incomplete. |

All eight handlers were written from scratch:

- handleReadCoils() — bit-packing, byteCount calculation, address validation

- handleReadDiscreteInputs() — same structure as read coils

- handleReadHoldingRegisters() — byteCount=qty*2, big-endian serialisation

- handleReadInputRegisters() — same structure as read holding registers

- handleWriteSingleCoil() — 0xFF00/0x0000 validation, coil write, echo response

- handleWriteSingleRegister() — address validation, register write, echo response

- handleWriteMultipleCoils() — bit unpacking, LSB-first, range validation

- handleWriteMultipleRegisters() — byteCount validation, big-endian parsing

## Bug #8 — RegisterMap Callback Deadlock

| Property | Detail |
| --- | --- |
| Location | src/RegisterMap.cpp — writeCoil(), writeHoldingRegister() |
| Severity | High — any callback that read a register would deadlock on the same mutex |
| Root Cause | User callbacks were invoked while holding mutex_. Any callback that called readHoldingRegister() would block waiting for the same mutex. |

// BUGGY: callback fired while mutex held

void writeHoldingRegister(u16 addr, u16 val) {

std::lock_guard lock(mutex_);

holdingRegisters_[addr] = val;

auto it = holdingWriteCBs_.find(addr);

if (it != holdingWriteCBs_.end())

it->second(addr, val);  // DEADLOCK if callback reads a register

}

// FIX: copy callback, release lock, then invoke

void writeHoldingRegister(u16 addr, u16 val) {

HoldingRegWriteCallback cb;

{

std::lock_guard lock(mutex_);

holdingRegisters_[addr] = val;

auto it = holdingWriteCBs_.find(addr);

if (it != holdingWriteCBs_.end()) cb = it->second;

}  // mutex released here

if (cb) cb(addr, val);  // safe to call any register map methods

}

## Bug #9 — OS-Specific Headers in Public Interface

| Property | Detail |
| --- | --- |
| Location | transport/include/UartTransport.h |
| Severity | Medium — prevented compilation on Windows or bare-metal embedded targets |
| Root Cause | <termios.h>, <fcntl.h>, <unistd.h> included in the public header. These are Linux-only POSIX headers. |

Fix: moved all OS-specific headers into UartTransport_Linux.cpp only. The public UartTransport.h now contains only the class declaration and includes only the portable IModbusTransport.h and <string>.

## Bug #10 — Preprocessor Macros for Type Aliases

| Property | Detail |
| --- | --- |
| Location | include/modbus/ModbusTypes.h |
| Severity | Medium — polluted global namespace; could cause unexpected symbol collisions |
| Root Cause | #define u8 uint8_t etc. are preprocessor substitutions with no scope. They affect every file in the translation unit, including system headers. |

// BUGGY:

#define u8  uint8_t   // replaces ALL occurrences of "u8" globally

#define u16 uint16_t

#define sz  size_t

// FIX: scoped type aliases

using u8  = uint8_t;   // respects C++ scope rules

using u16 = uint16_t;

using sz  = size_t;

PART V

Testing Strategy and Results

# 16. Testing Strategy

The test suite is structured as a pyramid: many fast unit tests at the base, fewer integration tests in the middle, and optional end-to-end tests at the top.

## 16.1 Test Pyramid

| Layer | Tests | Transport | What It Verifies |
| --- | --- | --- | --- |
| Unit: CRC | 14 | None | CRC algorithm correctness, byte order, known vectors, residue property |
| Unit: Codec | 42 | None | Frame encoding/decoding for all FCs, exception detection, CRC integration |
| Unit: Client | 36 | MockTransport | All 8 API methods, retry logic, coil value encoding, exception no-retry |
| Unit: Server | 38 | MockTransport | All 8 FC handlers, exception generation, broadcast, callbacks |
| Integration | 32 | LoopTransport | Live client↔server round trips, exception propagation, callbacks |
| TCP Codec | 35 | None | MBAP header encoding/decoding, TCP response parsing, exception in TCP |
| Total | 197 | — | All 197 pass with zero hardware required |

## 16.2 MockTransport

MockTransport is a test double for IModbusTransport. It stores bytes that were "sent" and allows injection of bytes that will be "received" on the next call:

class MockTransport : public IModbusTransport {

public:

void injectResponse(std::vector<u8> response);

const std::vector<u8>& lastSentBytes() const;

int  sendCallCount() const;  // for verifying retry count

// ...

};

// Example test:

auto* mock = new MockTransport();

mock->injectResponse({0x01,0x03,0x04,0x00,0x17,0x00,0x2B,0x0A,0x28});

ModbusClient client{std::unique_ptr<IModbusTransport>(mock)};

auto result = client.readHoldingRegisters(1, 0, 2);

assert(result.has_value());

assert((*result)[0] == 23);    // 0x0017

assert((*result)[1] == 43);    // 0x002B

## 16.3 LoopTransport (Integration Tests)

LoopTransport is a thread-safe blocking in-memory transport used in integration tests. Two instances are wired together: bytes sent to A arrive at B and vice versa:

struct LoopTransport : public IModbusTransport {

std::vector<u8>         buf_;

std::mutex              mtx_;

std::condition_variable cv_;

LoopTransport*          peer_;

Expected<size_t, ModbusError> send(span<const u8> data) override {

{ lock_guard lk(peer_->mtx_);

peer_->buf_.assign(data.begin(), data.end()); }

peer_->cv_.notify_one();

return data.size();

}

// ... receive blocks until peer sends ...

};

// Integration test usage:

Rig rig;  // creates wired server + client pair

auto result = rig.pump([&]{

return rig.client->readHoldingRegisters(1, 0, 3);

});

// pump() runs server.processOneRequest() in parallel

# 17. Demo Applications

## 17.1 modbus_server — Console Demo

The server demo initialises a register map with realistic industrial values, starts a Modbus server, and displays the live register state on the console:

Usage: modbus_server --mode uart --port /dev/pts/5 --baud 9600 --id 1

Output:

╔══════════════════════════════════════════════════╗

Holding Registers:

[0x0000] Motor Speed           = 1500.0 RPM  (raw: 1500)

[0x0001] Temperature           = 25.3 C      (raw: 253)

[0x0002] Temperature Setpoint  = 30.0 C      (raw: 300)  [writable]

[0x0003] Fault Code            = 0

Coils:

[0x0000] Pump Enable           = OFF

[0x0001] Alarm Reset           = OFF

Input Registers:

[0x0000] Supply Voltage        = 23.6 V      (raw: 236)

[0x0001] DC Bus Voltage        = 48.2 V      (raw: 482)

╚══════════════════════════════════════════════════╝

Press Ctrl+C to stop.

## 17.2 modbus_client — Console Demo

The client demo accepts a command-line instruction, connects to the server, executes one operation, and exits with code 0 on success or 1 on error:

# Read 3 registers

./build/modbus_client --mode uart --port /dev/pts/6 --baud 9600 \

read-regs --slave 1 --addr 0x0000 --count 3

Reading 3 holding register(s) from slave 1 starting at 0x0000...

Register[0x0000] = 1500

Register[0x0001] = 253

Register[0x0002] = 300

Completed in 12.4 ms.

# Write setpoint (35.0 C * 10 = 350)

./build/modbus_client --mode uart --port /dev/pts/6 --baud 9600 \

write-reg --slave 1 --addr 0x0002 --value 350

Writing 350 to register 0x0002 on slave 1...

Success (echo confirmed).

Completed in 8.1 ms.

## 17.3 End-to-End Workflow with socat

socat creates a pair of connected virtual serial ports on Linux, enabling end-to-end testing without physical hardware:

# Terminal 1: Create virtual serial port pair

socat -d -d pty,raw,echo=0 pty,raw,echo=0

# Prints: "N PTY is /dev/pts/5" and "N PTY is /dev/pts/6"

# The two ports are fully connected: write to 5, read from 6 (and vice versa)

# Terminal 2: Start Modbus server on /dev/pts/5

./build/modbus_server --mode uart --port /dev/pts/5 --baud 9600 --id 1

# Terminal 3: Client commands (all use /dev/pts/6)

# Read holding registers

./build/modbus_client --mode uart --port /dev/pts/6 --baud 9600 \

read-regs --slave 1 --addr 0x0000 --count 3

# Write a setpoint — watch server terminal update in real time

./build/modbus_client --mode uart --port /dev/pts/6 --baud 9600 \

write-reg --slave 1 --addr 0x0002 --value 350

# Enable pump

./build/modbus_client --mode uart --port /dev/pts/6 --baud 9600 \

write-coil --slave 1 --addr 0x0000 --value 1

# 18. Building the Project

## 18.1 Requirements

- Compiler: GCC ≥ 13 or Clang ≥ 17 (C++23 required for std::expected)

- Build system: Make (included) or CMake ≥ 3.16

- Platform: Linux (primary); architecture supports Windows and embedded

- Testing: No external test framework needed — tests use direct assertions

## 18.2 Build with Make

cd modbus-lib

# Build everything: library + tests + demo apps

make -j4

# Run all 197 tests

make test

# Build and run a specific test

make build/test_client && ./build/test_client

# Clean

make clean

## 18.3 Build with CMake

cmake -B build -DCMAKE_BUILD_TYPE=Debug

cmake --build build -j4

# Run all tests via CTest

cd build && ctest --output-on-failure

Appendices

# Appendix A — Project File Structure

modbus-lib/

├── include/modbus/          Public API headers (users #include these)

│   ├── ModbusClient.h       Client API with 8 read/write methods

│   ├── ModbusServer.h       Server API: start/stop/processOneRequest

│   ├── RegisterMap.h        Thread-safe 4-type data store

│   ├── IModbusTransport.h   Pure abstract transport interface

│   ├── ModbusFrame.h        Frame struct with helper methods

│   ├── ModbusError.h        Typed error hierarchy

│   └── ModbusTypes.h        u8/u16 aliases, FunctionCode/ExceptionCode enums

│

├── src/                     Private implementation

│   ├── CrcEngine.h/.cpp     CRC-16/IBM: calculate() and verify()

│   ├── ModbusFrameCodec.h/.cpp  RTU + TCP encode/decode

│   ├── ModbusClient.cpp     sendAndReceive + all API methods

│   ├── ModbusServer.cpp     Listen loop + all 8 FC handlers

│   └── RegisterMap.cpp      Thread-safe CRUD + deadlock-safe callbacks

│

├── transport/

│   ├── include/

│   │   ├── UartTransport.h  Portable header (no OS includes)

│   │   └── TcpTransport.h   TCP client/server factory methods

│   ├── UartTransport_Linux.cpp  termios + select() implementation

│   └── TcpTransport.cpp     POSIX socket implementation

│

├── tests/

│   ├── mock/MockTransport.h In-memory transport for unit tests

│   ├── test_crc.cpp         14 tests: vectors, byte order, residue

│   ├── test_frame_codec.cpp 42 tests: all FCs encode/decode

│   ├── test_client.cpp      36 tests: all API methods + retry

│   ├── test_server.cpp      38 tests: all handlers + exceptions

│   ├── test_integration.cpp 32 tests: live client<->server

│   └── test_tcp_codec.cpp   35 tests: MBAP encode/decode

│

├── examples/

│   ├── server/main.cpp      modbus_server demo app

│   └── client/main.cpp      modbus_client demo app

│

├── CMakeLists.txt           CMake build (all targets)

├── Makefile                 Simple make build

└── README.md                Build, usage, API reference

# Appendix B — Quick Reference: All Function Codes

| FC | Name | Request Bytes | Response Notes | Max Quantity |
| --- | --- | --- | --- | --- |
| 0x01 | Read Coils | 8 | ceil(qty/8)+5 bytes; bits LSB first | 2000 |
| 0x02 | Read Discrete Inputs | 8 | Same structure as FC01 | 2000 |
| 0x03 | Read Holding Registers | 8 | qty*2+5 bytes; regs big-endian | 125 |
| 0x04 | Read Input Registers | 8 | Same structure as FC03 | 125 |
| 0x05 | Write Single Coil | 8 | 8 bytes echo; value 0xFF00/0x0000 | 1 |
| 0x06 | Write Single Register | 8 | 8 bytes echo of entire request | 1 |
| 0x0F | Write Multiple Coils | variable | 8 bytes: addr+qty echo | 1968 |
| 0x10 | Write Multiple Registers | variable | 8 bytes: addr+qty echo | 123 |

# Appendix C — Exception Code Quick Reference

| Code | Name | Server Returns When |
| --- | --- | --- |
| 0x01 | Illegal Function | FC not in dispatch table (unknown operation) |
| 0x02 | Illegal Data Address | startAddr or startAddr+qty-1 exceeds valid range |
| 0x03 | Illegal Data Value | qty=0, qty>max, or coil value is not 0xFF00/0x0000 |
| 0x04 | Slave Device Failure | Hardware fault, unhandled exception in handler code |